Exercise 1

Course: 34373 Microcontroller Development for Advanced IoT using Embedded C

Made by: Hákon Hlynsson (s225765)

EX1: Look up the HAL_Delay() function in the HAL manual and explain what it does

According to the HAL manual (page 32), the HAL_Delay(Delay) function is used to pause code execution for a specific duration, defined in milliseconds by the input parameter Delay.

The function operates by relying on the SysTick ISR, which triggers every 1ms to increment a global tick variable. HAL_Delay() then continuously polls this variable, stopping the execution until the variable has increased by the specified delay value.

HAL manual:

HAL_Delay(). this function implements a delay (expressed in milliseconds) using the SysTick timer. Care must be taken when using HAL_Delay() since this function provides an accurate delay (expressed in milliseconds) based on a variable incremented in SysTick ISR. This means that if HAL_Delay() is called from a peripheral ISR, then the SysTick interrupt must have highest priority (numerically lower) than the peripheral interrupt, otherwise the caller ISR will be blocked.

EX2: Use the defined names found in gpio.c instead, and change frequency to 100ms.

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

while (1)

{

HAL_GPIO_TogglePin(LD2_GPIO_Port, LD2_Pin);	// Toggle LED

HAL_Delay(100);					//Delay 100ms

}

}

EX3: Look in HAL manual for another way to manipulate the GPIO (set/reset) and use this to make LED blink.

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

while (1)

{

HAL_GPIO_WritePin(LD2_GPIO_Port, LD2_Pin, GPIO_PIN_RESET);//Reset LED

HAL_Delay(100);							//Delay 100ms

HAL_GPIO_WritePin(LD2_GPIO_Port, LD2_Pin, GPIO_PIN_SET);	//Set LED

HAL_Delay(100);							//Delay 100ms

}

}

EX4: Create a variable X (char, int or uint8_t type) that counts up from 0. With each step the LED should blink X times (speed 100ms). Incorporate a pause of 1 sec between each increase of X. 0, 1, 2, 3, 4 etc blinks..

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

uint8_t X = 0;

while (1)

{

for (int i = 0; i < X; i++){

//breakpoint is place here fore next exercise

HAL_GPIO_WritePin(LD2_GPIO_Port, LD2_Pin, GPIO_PIN_SET);//Set LED

HAL_Delay(100);						//Delay 100ms

HAL_GPIO_WritePin(LD2_GPIO_Port, LD2_Pin, GPIO_PIN_RESET);//Reset LED

HAL_Delay(100);						//Delay 100ms

}

HAL_Delay(1000); 							//Delay 1000ms

X++;								//Increment variable

}

}

EX5: Use the Debug function to monitor and observe value of X and steps through code. Make screenshots.

Placed a breraking point after the i

EX6: Use CubeMX to configure one more GPIO to make an externally added LED blink. Upload code, screenshots from CubeMX and images of LED